| Agenda Item | 16.2 |
| --- | --- |
| Subject | Schedule of Works for Handyman |
| Author | The Clerk |
| Date | 30/01/2020 |

Overview

This document is setting out all of the previously resolved items of work to be carried out in the Parish.  Please note the Handyman is working on pricing up these works and a timetable for completion of the works.  These will be spread over the next coming months

Repairs to the playground areas – post walk around with Cllrs Monro and Horley.

- All sites to have

  - Relevant equipment sandblasted, rust removed and repainted. Please note all paint used must be lead free.

  - Play equipment to be cleaned.

  - All surfaces levelled underneath play equipment, by bins and benches and where required grass seed laid down – to note it may be necessary to removed matting, relevel the site and replaced the matting.

- Churchfields site

  - Remove the slide and all attachments to be dug out and cut/cover to make safe.

  - Remove wetpour from slide site, relevel ground and reseed with grass seed.

  - Repair works to the train including:

    - Removing all rust from metal work and scale back/repaint as necessary.

    - Cleaning down all woodwork and repainting with a non-slip brown stain.

    - Re-edging the slide.

    - Replace all four wooden steps with new wood.

    - Replace any missing bolts/screws.

  - Infill any gaps between wetpour in train area and grass or pavement.

  - Cut back any grass overgrown onto the wetpour.

- Recreation site

  - Remove balance beam and make site safe

  - Climber – Rota Webb – needs to be taken apart and cleaned/greased

  - Rotator Pole – bolt missing needs replacing

  - Remove shackles and bolts and repair/clean as necessary

  - Swing Basket – tighten connectors

  - Basket ball area – repaint Clerk will buy a new basket to put up

  - Remove single pole and place ‘No Dogs’ sign on large playground notice, replace the ‘No Horses’ sign onto the pedestrian fingerpost when installed

- Parish Field

  - Repair fencing as required

  - Replace worn connectors and chain links on swings as required

- Other Agreed Items

  - Replace noticeboard outside of Joan’s – Speak to Michael Horley

  - Repair, sand down, varnish wooden noticeboards x 2.  Remove right hand door and replace the inside with a suitable cork board/wooden panel for residents to pin posters on.  Remove the large wooden panel at the bottom with all old posters on it.

  - Support the Clerk in preparing site and choosing suitable replacement noticeboard opposite Londis stores.

  - Review broken seat at Westfield Lane bust stop (opposite entrance to Baldslow Down) and assess whether it can be repaired or replaced.

  - Remove memorial bench (at junction of Cottage Lane and A28) clear and leave site ready for new bench to put in place and clean paved area of debris and soil.

  - Sand down and varnish various benches within the Parish and repair as required.

  - Source and install traditional fingerpost for the recreation area/tennis club/bowls club once Clerk has received permission from Highways.