Agenda Item 8.2 – Summary of Playground Inspections ( please note recommendations are in the final column)

| Churchfield Play Area – Overall Area deemed LOW RISK |  |  |  |  |
| --- | --- | --- | --- | --- |
| Hazard Dog activity, children straying | Fences are not an automatic requirement for play areas but local conditions, such as proximity to roads or recorded dog activity in the area, should be taken into consideration | No risk value can be given | Risk asses the area using local knowledge and introduce controls as necessary | Consider putting up a fence around the playground area. |
| Hazard Trips | The pathway is cracked which may lead to trips | 2 x 2 low | Repair the path to eliminate the hazard | Speak to Optivo to see if they will improve the pathway |
| Outdoor Gym – Overall Area deemed MODERATE RISK |  |  |  |  |
| Hazard Signage | EN16630-7 Requires a sign in a conspicuous location containing at least the following information: This equipment is not play equipment Equipment is for use by youth and adults having an overall height or 1.4m Read and follow the instructions on the equipment Assurance about own medical safety | 3 x 4 Moderate | Fit a sign containing the relevant information | Still awaiting the right signage for this site. |
| Parish Field Playground – Overall Area deemed LOW RISK |  |  |  |  |
| Sports | The site is located near to a space where ball sports are played. Consideration should be given to orientation of pitches and approaches used by users of the area | 2 x 3 low | Monitor use and introduce control measures as necessary | Councillors may want to consider potential impact and if there has every been any accidents during matches. |
| Hazard No operator sign | BSEN1176 recommends signage with provision of the following information: general emergency telephone number, name of site operator and telephone number, phone number to contact maintenance personnel, name and address of the playground and any relevant local information, this may include recommended age range of users | 2 x 3 Low | Fit a sign including the relevant information | The Clerk will look to get a sign installed for the Parish Field. |
| Hazard Missing | The gate is missing | 2 x 2 Low | Replace the gate | Clerk to get an estimate to have the gate reinstalled or replaced. |
| Hazard Hard or sharp protrusion | The redundant gate latch is protruding and may cause injury | 2 x 2 Low | Remove the latch to eliminate the hazard | As above. |

| Hazard Damage | The wet pour repair is damaged, probably by tampering while still wet. This may affect the surfaces ability to protect users in the event of a fall | 2 x 4 Low | Repair the surface to eliminate the hazard | Recommendations to look at replacing these swings in the next few years with: - New play equipment. - Replacing the wetpour to match the newly repaired wetpour. |
| --- | --- | --- | --- | --- |
| Hazard Moss | The surfacing and swing seats have moss present from the adjacent trees | 2 x 2 Low | Ensure the surfacing an equipment is cleaned during routine maintenance |  |
| Hazard Free space | EN1176 2. 4.4.1. The swing seats are too close to the frame. The free space should be 20 percent of the length of the chains plus 200mm | 2 x 3 Low | Adjust the length of the chains to eliminate the hazard |  |
| Hazard Damage | The impact absorbent surface on the seats is deteriorating, possibly due to age or UV light | 2 x 2 Low | Monitor the damage and replace if the seat condition deteriorates or any metal frame is exposed |  |
| Recreation Ground – Overall Area deemed LOW RISK |  |  |  |  |
| Hazard Damage, sharps | Chain link is not considered an ideal fence for play areas. This fence has some damage and sharps at the link ends | 2 x 2 Low | Monitor use and consider alternative fencing if renewed | Look to install a natural hedgerow through a Lund grant. |
| Hazard Low branches | It is recommended tree branches are a minimum of 2m from the ground and do not enter the free space of equipment, to avoid climbing and injury | 2 x 3 low | Trim the tree branches to eliminate the hazard | Cllrs to consider a tree survey and review on all green spaces. |
| Hazard Full site address not given | BSEN1176 recommends signage with provision of the following information: general emergency telephone number, name of site operator and telephone number, phone number to contact maintenance personnel, name and address of the playground and any relevant local information, this may include recommended age range of users | 2 x 2 Low | Consider revising the signage for users contacting emergency services | The new green sign covers this bar the address of the site. |
| Hazard Worn surface | It is important to note that the critical fall height (CFH) and head impact criteria (HIC) may differ due to change of soil conditions, weather and local factors. It was not possible to test the grass matrix tiles on site, due to these factors. It is important to maintain good soil conditions to ensure the CFH is maintained. Due to compaction this surface may not meet the requirements | 3 x 3 Low | The surface should be routinely inspected, rubbish or debris removed. Check for compaction and consult the manufacturer /supplier for further advice on how to rectify | This is a recurring issue. Cllrs may wish to review the overall use of space and equipment to address these issues over the next few years. |
| Hazard Loose | The pole is slightly loose in the ground | 2 x 2 Low | Monitor for deterioration during routine maintenance and repair as necessary | As above. |
| Hazard Worn surface | It is important to note that the critical fall height (CFH) and head impact criteria (HIC) may differ due to change of soil conditions, weather, and local factors. It was not possible to test the grass matrix tiles on site, due to these factors. It is important to maintain good soil conditions to ensure the CFH is maintained. Due to compaction this surface may not meet the requirements | 3 x 3 Low | The surface should be routinely inspected, rubbish or debris removed. Check for compaction and consult the manufacturer /supplier for further advice on how to rectify | As above. |
| Hazard Life span | The exact age could not be verified. This type of swing frame is most likely over 20 years old and the structural integrity can no longer be guaranteed. It is recommended these swings are programmed for replacement. The risk assessment given is provisional | 3 x 4 Moderate. provisional | Consult records and programme for replacement | As above Cllrs may wish to look at revamping and reviewing the play equipment on the Recreation Ground. |